| News Letter. Schematic Request.; Sams-Club. Get Traffic.... <>... > National Repair Tips [1] 2 3 4 Many More Tips In Ours Members Section! No Limitations For Full Repairtips Members! 20.. To Remove Limitations, Click HERE! PLEASE CHOOSE A BRAND:; Top of Form Bottom of Form.. Our Store!. National NV370 - NV850 NATIONAL; NV370; 50 Hz Everywhere, No Video Distorted E.E.; 2200 µf 25vw O/c In P/s.. NATIONAL; NV370; Accepts Tapes Then Ejects.; Eject Load Belt Or Eject Switch's Vsm0048. NATIONAL; NV370; All Function Lights Are On Irrespective Of Operation.; Change Q6501,16501.. NATIONAL; NV370; Aud On Tuner Mutes When Afc Is Activated closing Tune Door; Check For Faulty C7105, 10µf 16v On Ch. Preset Pcb.. NATIONAL; NV370; Capstan Motor Oscillates Then Shuts Down.; Replace R1101 In Power Supply.. NATIONAL; NV370; Capstan Motor Oslilates And Then Turns Off No Functions; R1101 In Psu, R1105 O56 Fusable. NATIONAL; NV370; Capstan Motor Speed Varies; C2025,c2026,c2027 4.7µf/25v. NATIONAL; NV370; Capstan Plays Fast; Ic2002 Mn6168vif. NATIONAL; NV370; Chewing Tapes.; Replace Idler Pulley.. NATIONAL; NV370; Chews Tapes. Motors No Go.; Mode Switch. NATIONAL; NV370; Clock Display Ok. On/off Inoperative.; Replace Resistor R1001 On Power Supply Pcb.. NATIONAL; NV370; Counter Display Flashes B/t Dew And Normal Modes; Ic 6501 Front Pcb... NATIONAL; NV370; Dew Sensor On , Drum Spins All Time; Modify Dew Sensor With Earth Wire *. NATIONAL; NV370; Distorted Sound.; Change Ic4001 (ta7355pp) *. NATIONAL; NV370; Distorted, Hum Bars In Pic. In Playback Or E.E.; Replace Capacitor C1102 In Power Supply (up rating Voltage Rating Of Capacitor May Be A Good Idea).. NATIONAL; NV370; Drum Fails To Start. Low Torque; Drum Windings Faulty. Buy New Vcr! * *. NATIONAL; NV370; Erratic Mechanism Behaviour; Mode Switch Vss0091. NATIONAL; NV370; Function Display All Lite Up; Ic6501 Mn1250bvf2. NATIONAL; NV370; Goes Dead Interm led "on" Tape Counter Indic.Fails For Sec; Check 3.0v Zener Diode D1002 In The Regulator Circuit.. NATIONAL; NV370; Goes Into F.F. When Play Is Pressed.F.Rew. When Rec Is Selc; Change C6011 O/c On The Syscon Board.. NATIONAL; NV370; Int Failure To Retract Tape When Unloading.; Faulty Mode Select Switch Pt. No. Vss0110.. NATIONAL; NV370; Int Turning On And Off Completely.; Mains Transformer Windings Int. Replacement: Etp57pso1a.. NATIONAL; NV370; Interm Capstan Goes In Rev. Or Counter Counts In Rev.In P/b; Replace Mode Select Switch.. NATIONAL; NV370; Interm. Horiz. Twitching And Pulling On Playback.; "sticking" Of The Impedance Roller Up-stream Of The Full Erase Head. The White Roller And Its Insert Should Be Replaced.. NATIONAL; NV370; Intermittent Recording.; Change C3523 (near Base Of Q3501).. NATIONAL; NV370; Low Drum Torque. Interm. Drum Fails To Start Play.; Change Ic2005 (an6387) * *. NATIONAL; NV370; Low Gain On Receive.; Replace Q3.. NATIONAL; NV370; Low Gain Tv Pic.,hum Bar When Test Signal Is Switched On.; C1102 (2,200mfd) Leaky.. NATIONAL; NV370; Machine Goes Into Dew Alert Mode With No Dew Present.; Check Earth On Dew Sensor Mount And Tighten Screw.. NATIONAL; NV370; Motor Fails To Start. Tape Tangles.; Change Mode Switch.. NATIONAL; NV370; No Capstan Drive; C2020 * *. NATIONAL; NV370; No Capstan Drive.; Check An3822 Capstan Drive Ic Also .68ohm Resistor Not Shown On Cct It Feeds Pin 24 Of Ic, Occasionally It's A Wire Lik Marked R69.. NATIONAL; NV370; No Display; R1002 1O5 Fuseable. NATIONAL; NV370; No Display And No Functions.; Check R1001 O/c, Replace With .39 Ohms. Main Pcb Near The Rear Of Pcb (in Power Section).. NATIONAL; NV370; No Functions And No Display.; Check The -30v Line Is Missing. Replace Q1102, 30v Zener (d1109), R1102.. NATIONAL; NV370; No Go, Power Light On (dim) Ch Light (dim); R1 O/c Bottom Board O/c.. NATIONAL; NV370; No Go.; Mode Selector Switch.. NATIONAL; NV370; No Go.; Thermal Link Bottom Board O/c.. NATIONAL; NV370; No Go. No Power.; Cracked Cct Board (pcb Joins Bp201) 12v Zener S/c. *. NATIONAL; NV370; No Luminance. Flashing Colour In Display.; Change Vefy005 Pack.. NATIONAL; NV370; No Picture In Playback; D/j Q3562 Luminance Pcb.. NATIONAL; NV370; No Playback Colour.; Check Ref. Freq. Is Way Off Replace PCB. Module.. NATIONAL; NV370; No Power. Power Light Comes On, Goes Off.; Check Timer Rec Switch.. NATIONAL; NV370; No Record Or Playback.; C1102 Faulty.. NATIONAL; NV370; No Rf Output In Tv Or Video Modes; Check Q1101 For Dry Joints (no 12v To P702). NATIONAL; NV370; No Tape Functions. Drum Const. Spinning "d" In Clock Display.; Solder Wire From Earth Side Of Dew Sensor To A Convenient Earth Point On Machine (the Earth Shield On Tuner If Section Is A Really Good Spot).. NATIONAL; NV370; Noise Bar On Screen Every Six Seconds.; Change C2006 100 Mfd O/c. *. NATIONAL; NV370; Noise Bars Every 6 Sec; No Drum Sync C2006 100µf/6v. NATIONAL; NV370; Noisy Sound In Record Mode.; Change Ic4001,ta7355p.. NATIONAL; NV370; Not Loading Tapes In Play; Ic6003 Ba6209. NATIONAL; NV370; Not Recording; Qr6005 Dta124a Syscon. NATIONAL; NV370; Not Tunning Stations; 31v Rail Low Check C7103 .01µf. NATIONAL; NV370; Pb To Ee Picture Stays In Play; Qr6006 Dta124a Syscon * *. NATIONAL; NV370; Picture In Playback And E-e Greatly Distorted.; Change C1102 2200 Mfd 50v In Power Supply. *. NATIONAL; NV370; Poor Lumina, Chroma Flashes In Play; Vefy 005 Pack. NATIONAL; NV370; Poor Luminance In Pb; C3523 47µf/6v Luma Pcb. NATIONAL; NV370; Poor Pic, Large Drop Outs; C3504 100µf/6v Near Ic3501 *. NATIONAL; NV370; Remote Functions Not All Working.Eg..Play Not Working.; Remote Lead Has To Much Capacitance.Change Lead.. NATIONAL; NV370; Sometimes When Rew/f.F Is Selec. Machine Wont Function.; Change Mode Switch (ve50246) And Idler Arm Unit (vxpo521).. NATIONAL; NV370; Tape Jams With No Other Functions.; Check R1001 .12ohms For O/c.. NATIONAL; NV370; Tape Loads Up Runs Backwards...Chews Tape,counter Runs Back; Check Pin 39 Of Ic6001 Is Above 1v If So, Change Ic6001 And Ic2002.. NATIONAL; NV370; The Take-up Spool Doesn't Rotate In Play Mode.; Fit Modified Idler Assembly, Part No. Vxp0521. The Improved Idler Can Be Identified By The Blue Plastic Moulding Or Black Dot On The Left-hand Side Of The Idler Arm.. NATIONAL; NV370; Timer Resets To Zero,after Being Corrected,reset To Zero.; Change Clock I.C.. NATIONAL; NV370; Video In Play And Ee Distorted Bad; C1102 2200µf/50v Psu *. NATIONAL; NV370; White Pulses On Replay; Check Screw In Lower Pcb Head Amp Section. NATIONAL; NV370; Will Not Inject Cassette.; Check Switches On Cassette Carriage Vsm0046. *. NATIONAL; NV370; Wobbling Picture With Slight Interference On Sound.; Check Tape Is Not Rubbing Against Washer At Bottom Of The Impedance Roller.. NATIONAL; NV370; Wow & Flutter. Motor Torque Low; Ic 2004 An3822, Capstan Stator. Vek2129. NATIONAL; NV450; Clock Ok But No Operate.; F1102 O/c No 12v Unswitched Or 18v Unreg.. NATIONAL; NV450; Display Goes Weird, Flashes Weird Numbers, Or No Display.] rest Of Machine May Work.; Replace Crystal On Top Tuner Adjust Board. x1 Near Ic 7501.. NATIONAL; NV450; Hum On Ee.Video In B/w; Power Supply Caps C1104 And C1103. NATIONAL; NV450; Loads Up, Plays For Few Secs. Stops And Unloads.; Faulty Photo Interupter Under T/u Spool Should Switch At +5 Volts.. NATIONAL; NV450; No Display, No Operation. Reset Line Pulsing, No Switched 12.7v, Unreg 45v Noisy.; C1102 100uf 63v Cap O/cct. NATIONAL; NV450; No Mechanical Reset At Vtr On; Check Tf2001, Temperature Fuse, P.No. Eyp2br163g Mounted On Capstan Drive Ic,an3821k. NATIONAL; NV450; No Record Or Playback.; Rf Converter Faulty.. NATIONAL; NV450; Picture Wanders Horiz. And Music Sounds Awful.; Check Setting Of Servo Motor, Control Track Clean, Vibrating Exit Tape Guide, Or Mechanical Resonance In Audio Control Assembly.. NATIONAL; NV450; Plays B&w And Ee Noisy, Or Hum Bar Through Picture.; C1002 O/c (100uf 35v). NATIONAL; NV450; See Nv250. NATIONAL; NV450; Slow Ff/rw, Fsearch Drops Into Ff, Rsearch Drops To Rewind; Replace 1c2003 (mn6168vif). NATIONAL; NV450; Stops Intermittently Or When Warm In Play Or Record.; Check Tf2001, Temperature Fuse, P.No. Eyp2br163g Mounted On Capstan Drive Ic,an3821k. NATIONAL; NV450; Tape Plays /rec. For A While Then Stops. Counter Interm .; Replace Reel Sensors.. NATIONAL; NV450; Vtr Switches Off By Itself Or Won't Switch Off Remote Fault; Check Function Of Ic 7504, Mn14531vtv. NATIONAL; NV450; Won't Play, No Capstan Motor Fwd,capstan Reverse Ok; Check Ic 2002,an3821k On Main Pcb. Pin 11 Should Be Low In Play, High In Rewind. NATIONAL; NV450; Wont Accept Tape. Wont Fully Eject Tape.; Replace Worm Wheel On Eject Assy.. NATIONAL; NV470; Colour Drops Out, Hum Bar In Colour; C1002 O/c ( 100uf,25v). NATIONAL; NV470; No Vtr No Display; Reg 12v Low. D1001-d1004 O/c. NATIONAL; NV470; Powers-up Clock Ok Accepts Tape But No Functions.; Diln59 Diode Led For Sensors.. NATIONAL; NV470; Tv Pic Snowy With Vcr Operate Off; 12v On Pin8 Of P1101 R/s , D1108 13v Zenner. NATIONAL; NV600; Cannot Set Clock Or Program Modes; Sw 7504 Esd14182. NATIONAL; NV600; Capstan Fails To Start In Play Or Record Modes.; Change O/c R2031, 100 Ohm.. NATIONAL; NV600; Capstan Speed Fast; R2080 12k C2048 100µf. NATIONAL; NV600; Capstan Speed Faulty; C2038 10µf/16v. NATIONAL; NV600; Capstan Speed Too Fast On Playback.; Check Value Of R2080, 12k Ohms, Or D/j On C41.. NATIONAL; NV600; Chews Tapes In Record Pause Mode; Pinch Roller Clearance ,adjust Screw. NATIONAL; NV600; Drum Int Stops Or No Start; C2023,c2024,c2025 .22µf/50 Tant Cap. NATIONAL; NV600; Drum Motor Intermittently Fails To Start In Play Or Rec Mode Or Wont Start At All, Or Has Low Torque; Suspect Ic 2001, Upc1405c,in Servo Circuit.. NATIONAL; NV600; Drum No Go; Ic2002 An6677. NATIONAL; NV600; Ejects After 8 Seconds And No Functions.; Change D6025. Ma165.. NATIONAL; NV600; Fails To Start After Entering Play Or Record Mode.; Check O/c R2059, 6.8ohm 1w. Or Change Ic2002 (an6677).. NATIONAL; NV600; Interm. Capstan Speed.; Dry Solder Joins On Capstan Motor Drive Transistors Q18, Q6017 And Q6013 On Main Pcb Servo Section.. NATIONAL; NV600; Low Audio In Ee; C759 10µf. NATIONAL; NV600; Luminance Or Playback Fades After 10 Sec.; Change Ic3002, (an6337).. NATIONAL; NV600; No Chroma.; D8004 Leaking.(ma165). NATIONAL; NV600; No Clock Or Channel Select.; Change Ic7504, Mn1228 Or Ic7505, An5033.. NATIONAL; NV600; No Rew. Or Ffd.; Check D1506 And D1505.. NATIONAL; NV600; Not Selecting Stations.; No 28v To Pin G Of Ic7504, Replace O/c R7547, 100k Ohms.. NATIONAL; NV600; Not Storing Stations; Missing 28 V Ic7504 Check R7547 100k. NATIONAL; NV600; Not Tunning Stations; R7543 180O. NATIONAL; NV600; Plays Slow Audio, No Picture. Audio Buzz In Ee No Picture.; Q1003 Dry Joints. NATIONAL; NV600; Poor Colour In Playback.; 3300uf/16v Cap In Power Supply. NATIONAL; NV600; Poor Luminance In Pb; Ic3002 An6337. NATIONAL; NV600; Poor Picture From Self Record; C2004 10µf/16v Or Heads. NATIONAL; NV600; Poor Picture In Play.Drum Speed; C2024 .22µf/50v. NATIONAL; NV600; Poor Picture In Still And/or Cue Modes.; Relay Ry3001 Dirty Contacts On Main Board Under Shield On Main Board 9 Volt Dp Dt Mini Relay See Wes Catalogue. NATIONAL; NV600; Power Stays On 10 Seconds Then Power Off.; Check Cassette Carriage Position Switches.. NATIONAL; NV600; Tangles Tapes In Search And Review Modes.; Check R2096, 39 Ohms In Capstan Brake Circuit.. NATIONAL; NV600; Tapes Tangle In Cue Or Review; R2096 39O Fusiable. NATIONAL; NV600; Tuning Faulty.; Replace R7543, 180 Ohms.. NATIONAL; NV600; Vcr Stays On Few Sec Then Off; Check Carriage Switches. NATIONAL; NV600; Wont Rewind . Locks Up Take Upreel; Replace Brake Pads On Brake Arms , The Spring Loaded One. NATIONAL; NV7000; Black Pic In Pause And V/search; Q6709 False Vert Sync C/t. NATIONAL; NV7000; Clock Display Segments Wrong But Still Programs Correctly.; Replace Three Driver Ic's All Icta57.. NATIONAL; NV7000; Clock Display Wrong.; Replace Driver Ic's (3) Ta57. NATIONAL; NV7000; Digits In Clock Go Out,days Go Brighter When Mode Is Selec.; Check Ha1780 6v Regulator Ic501 And 12v Line If Ok Check For D/j's On Timer Panel, And Steady 262khz On Pin 40 Of Ic7505.. NATIONAL; NV7000; Int Speed Search; Shaft On T/u Clutch Too Tight C&lub. NATIONAL; NV7000; No Capstan Lock; C2040 47µf/16v. NATIONAL; NV7000; No Clock Display; Q7501 2sc1683 , D7501 O/c. NATIONAL; NV7000; Noise On Screen Cannot Adjust Pg; Ic2001 An6350. NATIONAL; NV7000; Noisy Shuttle Search.; Replace Or Readjust Mode Switch.. NATIONAL; NV7000; Pinch Solenoid Wont Pull In.; Clean Capstan Shaft.. NATIONAL; NV7000; Poor Tv Reception Through Vcr And In Ee Mode.; Replace All 3 Transistors In Rf Unit.. NATIONAL; NV7000; Snowy Picture.; Rf Amp Earth Loose Tighten Screw At Top Of Can, Change Q3 In Amp.. NATIONAL; NV7000; Will Not Vtr On.; No Sw. 12v At Pin 1p1003. Change Q1003, Use 2sc1847 and Zener Diode D1015 13v.. NATIONAL; NV7200; Drum Speed Faulty; C2030,c2031,c2032 .22µf/50v. NATIONAL; NV7200; In Timer Mode Rec. Led Flashes, Tape Laces Up,runs Spasmod.; Change Microcomputer I.C.. NATIONAL; NV7200; Int Failure To Record.; Check If 9v On Pin 9 Of Ic3001 If So Check Q3003.. NATIONAL; NV7200; Int No Play. Usually After Lace Up Then Unloads.; Check For Bad Contacts On The Cassette Detect Leaf Switch.. NATIONAL; NV7200; Int Operation Especially In Timer Mode.; 1: Mechanics Still, Sliding Bars Need Oiling (both Sides.) 2: Mechanic Sensor Switch Faulty Contacts. 3: Check Leading Belt.. NATIONAL; NV7200; Int Play Mode Table And Reel Jamming.; Change Supply Reel Table Unit.. NATIONAL; NV7200; No Functions; R1019 120O O/c. NATIONAL; NV7200; No Reel Drive; Q6046 2sd973. NATIONAL; NV7200; No Timer Programming.; Check Clock Operation If Ok Check Program Switch For Bad Contacts.. NATIONAL; NV7200; Noisy Shuttle Search.; Readjust Mode Switch Or Replace.. NATIONAL; NV7200; Pinch Solenoid Wont Pull In.; Clean Capstan Shaft.. NATIONAL; NV7200; When Play Pushed,clock Loses Time And Display Goes Haywire.; C1005 On 13v Rail.. NATIONAL; NV7200; Wow In Sound. Both Record And Play Modes.; Check Tp2009 For Phase Lock Drift If Present Check Capstan Motor For Tight Bearings, Also Back Tension, If Ok Change Capstan Motor.. NATIONAL; NV730; Changes B/t Lp To Sp On Lp Tapes; Clean Control Head. NATIONAL; NV730; Chews Tapes; P5 Arm Unit Seized Vxl1223. NATIONAL; NV730; Chews Tapes Int; Dj On Power Transistor Q1504 On Heatsink Rhs Rear.. NATIONAL; NV730; Display Faulty; X7504 Vsx0138. NATIONAL; NV730; Faulty Inject Or Eject. Vtr Turns Off.; Replace Gear On Cassette Carriage P. No. Vxp0604.. NATIONAL; NV730; Flag Waving; Check Exit Guide Vxp0444. NATIONAL; NV730; No Backup Memory; Check "gold Caps". NATIONAL; NV730; No Capstan Drive; Ic2004 An3822, Or Capstan Mtr Vek2208. NATIONAL; NV730; No Capstan Drive In Play Or Record Modes.; Check R2094 And R2054. If Ok Replace Ic2004, An3822k With An3821k.. NATIONAL; NV730; No Drum Int; C2018 3.3µf. NATIONAL; NV730; No Drum Spin; Ic2003 An6387. NATIONAL; NV730; No Go; Ic 6004 Ba6209 Load Mtr Ct.. NATIONAL; NV730; No Go Clock Ok; Ic6001 Mn15342vgd, X6001 Vsx0082, Ic7504 Mn1228. NATIONAL; NV730; No Go F1003 O/c; Ic 1501 Stk5361l R1505 68 O Fusible. NATIONAL; NV730; No Go Missing 30 And 45 V Rails; R1002 4O7. NATIONAL; NV730; No Go No Clock Display; R1002 4O7. NATIONAL; NV730; No Go No Clock Display; Ic7503 Mn1512vtj. NATIONAL; NV730; No Or Intermittent Reel Motor Drive.Eject, Tape Caught Cyl.; Check For D/j On Power Transistors, Q1504, Mounted On Heatsink.. NATIONAL; NV730; No Reel Motor Function.; Resolder Q1504 And Bp1504.. NATIONAL; NV730; Not Eject Tapes Or Accept; Worm Drive Vxp0575. NATIONAL; NV730; Pic Flutters; Entry Guide Vxp0444. NATIONAL; NV730; Tapes Crease In Cue Review; Wear In Capstan Bearing. NATIONAL; NV730; Turns Off When Ejecting; Carriage Switches Vsm0050. NATIONAL; NV730; Vcr Turns On Then Off 4 Sec; Carriage Switches. Vsm0050. NATIONAL; NV730; Vtr On, Rewinds For 4 Sec. Then Turns Off.; Check Position Of Cassette Carriage Position Switches.. NATIONAL; NV730; When Play Mode Selec. After 1 Hr. Unlaces And Turns Off.; Change Ic2004 See Fault In This Model.. NATIONAL; NV730; Wont Memorize Stations; Ic7504 Mn1228. NATIONAL; NV730; Wont Play. Not Lacing Up; Mode Switch Ves0091. NATIONAL; NV730; Wont Switch On Not Accepting Tapes; Ic6005 Ba6209. NATIONAL; NV777; Accepts Command With No Tape In Unit. Rew Command Ok.; Check Main Cam Plastic Cog For Missing Teeth. (set Mode Switch For Stop Not Eject).. NATIONAL; NV777; Capstan Fast; Ic2008 Or C2047-2049. NATIONAL; NV777; Capstan Runs On From Cue Chews Tapes Int?; C2047,c2048,c2049 .47µf/50v Rl6409 *. NATIONAL; NV777; Chews Tapes; P5 Arm Seized Vxl1047. NATIONAL; NV777; Drifts Off Tune; Mn1228. NATIONAL; NV777; Drum Motor Fails To Start. Low Drum Torque.; Replace C2059, C2060 And C2061, 0.22µf 50v. Also 2.2 µ Caps On Same Ic. NATIONAL; NV777; In Play,cue Mode When F.Fwd Button Released Play Wrong Speed; Replace C2047, C2048 And C20249, 0.33µf 50v.. NATIONAL; NV777; Int Chroma Lines In Pb; Dj On Ic3001 Thickfilm. NATIONAL; NV777; Int Power To Vtr; Power Transformer Int. NATIONAL; NV777; Jitter In Pause Mode.; Rubber Plug On R.H.S Of Machine Gives Access To Present Adj.. NATIONAL; NV777; Lack Of Line Lock.; Check Whether The Drum Pg Pulse Is Present At Tp2006. If Not, Replace The Drum. If The Pg Pulse Is Present, Check Switching Pulse Is Present At Tp2011. If Not,suspect Ic2003 (an6346n). If Switching Pulse Is Ok Check The Squarewave Output At Pin 27 Of Ic2. NATIONAL; NV777; Looks Like Faulty Heads, One Head Only At Rf Test Point.; C3034 Leaky (10uf/25v). NATIONAL; NV777; No Capstan Drive; C2021 10 µf/16v. NATIONAL; NV777; No Clock Or Screen Functions.; R7560 Thermal Resistor On 40 V Rail. Located On Timer/operation Pcb. NATIONAL; NV777; No Functions Wont Switch On.; Replace Reset Capacitor On Ic6001.. NATIONAL; NV777; No Go But Clock Ok; Mode Switch Vss0060. NATIONAL; NV777; No Playback Or E.E. Sound Lcd Counter Display.; Check Q6062 (2sc1847) On Syscon Board O/c Bd137 Ok.. NATIONAL; NV777; No Reel Motor Drive; Q6016,q6017 Or Ic6007 Sta131a. NATIONAL; NV777; Slow Or Fast Capstan When Hot?; Ic2001 Mn6168via. NATIONAL; NV777; Slow Or Fast On Playback. Often Only When Hot.; Replace Ic2001, Mn6168.. NATIONAL; NV777; Tape Plays /rec. For A While Then Stops. Counter Interm .; Replace Reel Sensors.. NATIONAL; NV777; Tv Jitter In Pause.; Adjust Jitter Pot.. NATIONAL; NV777; Vtr And Clock On,wont Inject Tapes, No Mech. Functions.; Check Position Of Mode Switch. P. No. Vss0060.. NATIONAL; NV777; Wont Tune Channels.; R7019 O/c On Tuner Board.. NATIONAL; NV778; See Nv777. NATIONAL; NV788; See National Nv777. NATIONAL; NV830; No Capstan Motor Drive In Playback, Rew Or F.Fwd.; No 12v On Pin 24, Ic4. Repair Fusible Link W313.. NATIONAL; NV830; No Colour.; Check 12v Rail, Also Q1003 And D1004 For Leakage.. NATIONAL; NV830; No Tuner Picture.; Check Transistors Q7006 And Q7007.. NATIONAL; NV830; One Audio Channel Low.; Replace Q4315, 2sd655.. NATIONAL; NV830; Overloading On E-e.; Change Bn5115b I.F. Chip.. NATIONAL; NV830; See Nv370. NATIONAL; NV830; Will Not Tune Stations.; Check/replace Ic7503.. NATIONAL; NV850; All Displays Light Up For Approx,8secs Then Vtr Off.; Suspect Load Motor And Ic Driver Ic 6003, Ba6209 In The System Control Section Of The Main Pcb. NATIONAL; NV850; At Vtr On Or Power On,capstan Motor Rotates No Mech Reset; Suspect Load Motor And Ic Driver Ic 6003, Ba6209 In The System Control Section Of The Main Pcb. NATIONAL; NV850; Blank Screen In E-e Tuner Mode,audio,ok. Cassette Play Ok.; Check For Faulty Tr703, 2sd636, In Demodulator Unit. NATIONAL; NV850; Int Capstan Stops Or Vaires; Pressure Roller Seizing. NATIONAL; NV850; Int Drum Stops In Pb/rec; Ic2005 An6387. NATIONAL; NV850; No Go; R1001 O34 Fuse Or C1101 3300µf/16v Or Ic6003 Ba6209 Loadmtr. NATIONAL; NV850; No Picture In Ee; Q702 2sa564. NATIONAL; NV850; No Test Sig; X3001 Vsx0099. NATIONAL; NV850; Red Vtr Indicator On All Times Clock On But Can't Be Set; Check For Open Circuit R1001 In The Power Section Of The Main Pcb.. NATIONAL; NV850; See Nv370. NATIONAL; NV850; Sound On HIFI Ok? Normal Varies; Ic2004 An3822- Use An3821 As Replacement. NATIONAL; NV850; Unable To Tune Stations; Check Ic7505 If Unable To Change Tuning Of Station, Else Replace Ic7503.. NATIONAL; NV8600; Flashing On Screen With Varying Brightness In E.E. And Rec.; Check The Record/playback Switches If Ok Look For D/j's In The R.F/i.F. Stages (monitor Tp301 On Luminance Panel.). NATIONAL; NV8600; Goes Abnormally Fast In Play.; Check 2sa699 S/c Also Capstan Motor Should Draw (30 Ma Off Load 80-120 Ma On Load). Any More Than This Means Capstan Motor Faulty.. NATIONAL; NV8600; Playback Of Pre-recorded Tapes Ok Not Rec Luminance; Check For Faulty Q301,2sc828 On Luminance Process Pcb. NATIONAL; NV8600; See Nv2000. NATIONAL; NV8600; Wont Play Or Record. Pinch Roller Wont Engage.; Check Solenoid Pull In Transistor For O/c Replace With Bd 139, Also Check Damping Diode D624.. NATIONAL; NV8610; Drum And Capstan Motors Running As Soon As Power Is On.; Check Or Replace Loading And/or Load End Switch. (mounted On Sub Board Vjb00310 On The Deck.). NATIONAL; NV8610; See Nv2000. NATIONAL; NV870; Faulty Loading Motor Drive.; Replace Ic6004, Ba6238.. NATIONAL; NV870; Faulty Loading Motor Operation; Ic6004 Ba6238. NATIONAL; NV870; No 12v B+ On Tuner.; Replace L2002, 1 Mhz.. NATIONAL; NV870; No 14v To M. Comm. Motor.; Check F2001, Vsf0035.. NATIONAL; NV870; No Go, No Display Or Jumbled; Ic7501 Mn1455bvl. NATIONAL; NV870; No Tuning Memory With Dim Display.; Check - 28v To Memory Chip Via R7550 (27ohm).. NATIONAL; NV870; Not Recording Audio On One Channel; Replace C4594 And C4565, Both 47µf 6v Capacitors.. NATIONAL; NV870; Vtr Turns Off 10 Sec, Wont Accept Tapes; Dj On Q1503 (h/sink Rear Corner). Now you can find us faster by using our new Browser Toolbar ........................MORE.; Live Help close window.. Im ready to assist!. Testimonials. RESERVED FOR AD.. RESERVED01; RESERVED02.. Our Forum; Electronics Troubleshooting; Fletchermanuals.com; Fletchershopping.com; Our Blog; Site Map; Contact-Us. |  |  |
| --- | --- | --- |
| News Letter. Schematic Request.; Sams-Club. Get Traffic.... <>... > National Repair Tips 1 [2] 3 4 Many More Tips In Ours Members Section! No Limitations For Full Repairtips Members! 18.. To Remove Limitations, Click HERE! PLEASE CHOOSE A BRAND:; Top of Form Bottom of Form.. Our Store!. NV100,NV280,NV1000.. NATIONAL; MV300; No Tracking Control In Playback.; Q6003 O/c Replace Bc339. NATIONAL; MV300; Not Recording Video Or Audio; Check Non Record Supply. C3020 220µ F 6.3v S/c Q3020.. NATIONAL; N788; All Functions Work Except Eject.; Replace Ic 6001 Mn15342vgd. NATIONAL; NV100; All Functions Stop After Several Seconds.; As Above, Only Check Photo Sensors Under Take-up Reel.. NATIONAL; NV100; Capstan Servo Slow/oscillating; Dry Joint Plated Thru Hole Joining R2024 To R2042/c2020. NATIONAL; NV100; Capstan Speed Varies.; C2017 (10uf), Q2003 Servo C/t. NATIONAL; NV100; Drum Int Fast (when Cold?); C2029 47uf Near Ic2003. NATIONAL; NV100; F.F. And Rew. Stop After Several Seconds, Play, Rec. Ok.; Check Function Of Photo Sensor Under Supply Reel Table. P.No. On2160. Possibly Only Cleaning Of Both Sensor And Underside Of Reel Is Required.. NATIONAL; NV100; Int Stops All Functions; Reel Sensors Prt# On2160, Clean First.. NATIONAL; NV100; Interm. Capstan Speed Varies. Possibly When Cold.; Replace Suspect C2017, 10µf 16v, Off Pin 3 Of Ic 2005.. NATIONAL; NV100; Interm. Drum Speed Is Very Fast. Possibly Only When Cold.; Replace Suspect C2029, 47µf 6v, Near Ic2003.. NATIONAL; NV100; Intermittent Stopping; Check Microcomputer Port Expansion Chip 16003.. NATIONAL; NV100; No Colour Rec/play; C8009. NATIONAL; NV100; No Pic Or Audio In Ee; Q733. NATIONAL; NV100; No Playback Colour For First Few Seconds.; (depends On How Long Machine Has Been Switched Off After Overnight Rest 3 Mins.) Replace C8009 Near Ic8001.. NATIONAL; NV100; No Power No Go; F6501 T2a. NATIONAL; NV100; No Sound At Turn On, Then Sound Appears Slowly.; C4008 In Sound Section Leaky. 22uf 10v. NATIONAL; NV100; No Video (int) In Playback; D/j On Ic3001. NATIONAL; NV100; Not Erasing Previous Video/audio; Q1012 2sb643 Ic1002 Vcr0044. NATIONAL; NV100; Not Recording Colour; Ic8002. NATIONAL; NV100; On Tuner No Display; C6763 10uf Reset Pcb. NATIONAL; NV100; Playback Audio; Check Voltage At Pin 1 Of Ic4001 If Low (2vo Instead Of (4.3v) Replace C4016.. NATIONAL; NV100; Takeup Idler Not Turning In Play. Ff & Rw Ok; Remove And Clean Play Idler Gear Assy (transmit Arm Unit.. NATIONAL; NV100; Turns Off After 10 Sec. Stop Led Flashes On Eject; R6402 O/c May Need Ic6401 & Load Mtr. NATIONAL; NV100; Turns Off After Several Seconds, No Reset Mechanical; R6421 1k Ic6401 If 12v Ok On Pins 2,3 Ic6401. NATIONAL; NV100; Unit Wont Go Into Full Load Pos. After R6401 Has Failed.; Replace Only With .47 Fusible Resistor. Loading Mechon.. NATIONAL; NV100; Vtr Turns Off After Several Sec. No Mech. Reset At Vtr On.; Check R6421, 1k Ohm. Suspect Ic6401, Vcr0045, If Supply 12v O.K. At Pins 2 And 3 Of Ic 6401 And No Output To Motor On Pins 8 And 12.. NATIONAL; NV100; Wont Pause.; Is The Front Panel Pluged In. Ie Slow Tracking Control. NATIONAL; NV180; Picture Faulty Due To Constantly Changing Cylinder Speed.; Check For Dry Joint On Capacitor C46, Mounted On Trackside Of Main Pcb Under Ic2, Near Drum Motor Connector.. NATIONAL; NV180; Snowy Pic. Looks Like One Dirty Video Head.; Check For Faulty Or Dry Joint C2531, 0.1µf, Off Pin 22 Of Ic 5 On Main Pcb.. NATIONAL; NV180; Wont Go Battery Charge Mode When Conn. To Vw-a18 Charger.; Check For Open Circuit Track On Main Pcb Between: Pin 2 Of P215, Rear Right=hand Corner; And Anode Of D6501, Front Right Ahnd Corner. Track On Component Side Of Pcb Too Thin To Carry, Use Wire Link To Join These Two Points.. NATIONAL; NV180; Wont Rewind. F.F. For Only A Few Sec.; Replace Reel Stator Base Unit Vxa2045.. NATIONAL; NV2000; Capstan Motor Fails To Start After Lace Up.; Check .5v On T.P. 2008 In Playback. Replace Q2015 - use 2sc1826.. NATIONAL; NV2000; Clock Failure With No Display; Check 100 Ihm Safety Resistor (r7632 In Nv 366), Check Back Up Battery Is Not Leaking.. NATIONAL; NV2000; Clock Wrong Resets.; Ic7501 (mn1435vx).. NATIONAL; NV2000; Display Slow In Coming On, Disappears When Switched On.; Replace C1009 (1000 Mfd).. NATIONAL; NV2000; Distorted Pic In Pb,no Test Sig,no Clock; C1009 Replace. NATIONAL; NV2000; F/f When Rewind Selected. Plays Fast; 9v Reg U/s, Q6012, Q6013 Syscon. NATIONAL; NV2000; Good Picture On Playback Poor On Record.; Suspect Video Head.. NATIONAL; NV2000; Head Speed Varies, Switches To Standby After Warm Up.; H.S.W.P. Disappears, All Other Waveforms On Ic701 Correct, R772 Was 47 Ohm, Change To 470 Ohms.. NATIONAL; NV2000; Int Colour Recording When Hot.; Loss Of Afc Sync (afc Osc Tp505 Not Synced With Video Signal Tp509). Replace Ic503 And Adjust Afc.. NATIONAL; NV2000; Int Jams Up.; Replace Mode Switch Or Load Belt.. NATIONAL; NV2000; Int Tracking Head Speed Too High.; Very Low Fg Signal, Fg Coil Int Contact, Replace Lower Head Drum Assembly.. NATIONAL; NV2000; Load Motor Wont Return Mode Switch To Correct Position.; Replaced D6010.. NATIONAL; NV2000; No Audio Record.; No Supply To Bias Osc. (q617 S/c). NATIONAL; NV2000; No Clock - No Functions.; Q1007 In P/s.. NATIONAL; NV2000; No Picture Recording.; O/s F1201.. NATIONAL; NV2000; No Play/ff/rew.; Q6004 O/c.. NATIONAL; NV2000; No Playback, No Sound.; Replace Ic801 If Pin 35 Permanently High.. NATIONAL; NV2000; Not Tracking, Head Speed Too Fast.; Ic704 Faulty (s/c). NATIONAL; NV2000; Picture Off Tape Unstable.; Back Tension Band Pad Faulty.. NATIONAL; NV2000; Pinch Solenoid Wont Pull In.; Readjust Mode Switch.. NATIONAL; NV2000; Play Mode Then Stops. Capstan Wont Operate In Play/rec.; Replace Both Loading Belts.. NATIONAL; NV2000; Poor Tv Reception Through Vcr And In Ee Mode.; Replace All 3 Transistors In Rf Unit.. NATIONAL; NV2000; Servo Slow To Lock In colour, sound Variation When On.; Drum Or Capstan Lock Out Of Adjustment.. NATIONAL; NV2000; Slow Capstan Speed Poor Tracking.; Replace C1009 In Power Supply.. NATIONAL; NV2000; Slow Capstan, Hum On Video.; C1009 1000 Mfd 35v Faulty.. NATIONAL; NV2000; Snowy Picture.; Rf Amp Earth Loose (top Of Can) Tighten Screw.. NATIONAL; NV2000; Stops For No Reason After A Long Period Of Time.; Int Reel Motor - Try Clean And Lube Otherwise Replace.. NATIONAL; NV2000; Tape Gets Stuck In Housing.; Replaced D6010.. NATIONAL; NV2000; Tape Jammed In Unit.; Bad Crimp Connection On Plug.. NATIONAL; NV2000; Tape Jams In Housing, Mode Select Switch Wont Go to Neutral Pos.; Replace D6010.. NATIONAL; NV2000; Tape Not Rewinding Into Cassette.; Reel Idler Slipping (clean). NATIONAL; NV2000; The Timer Will Not Program.; Change Clock I.C.. NATIONAL; NV2000; Timer, Demod, capstan Motor 18v Low When Vtr Switched On.; Collector Of Q1007 Drops In Value. Change C1009 And/or leaky D1011.. NATIONAL; NV2000; Tracking - Noise Bars.; Loading Arm (b) Not In Location - Double Acting Spring Loose, Reinsert Spring.. NATIONAL; NV2000; Vtr Indication On At All Times.; Short In 6v Line. F1003 O/c Ic1501 In Power Supply Faulty. Zenner Diode 6.8v. D7542 On Timer Pcb, Short Or Leaky.. NATIONAL; NV2000; Wont Tune Sound, Pic. Together. No Sound On Prerecor. Tape.; Change C4019 (o.33mfd) O/c.. NATIONAL; NV250; After Goes Into Play Mode Stays Laced Up But Stops.; Replace Mode Select Switch. (to Be Done In Eject Mode And Align V Notch).. NATIONAL; NV250; At Power On Capstan Rotates For Approx 6 Sec.,turns Off.; Change Ic6003 (ba6238). NATIONAL; NV250; Capstan Wont Start When Laced Up For Play Or Record.; Faulty Mode Switch.. NATIONAL; NV250; Capstan Wont Start. Turns Off; Mode Switch Vss0110. NATIONAL; NV250; Clock Erratic Wont Flash; Q6001 2sb636. NATIONAL; NV250; Display Fully Lite; Ic6501 Mn15823vtt. NATIONAL; NV250; Drum Stops And Vcr Turns Off.; Faulty Mode Switch.. NATIONAL; NV250; E-e Tuner Mode Ok, On Playback Blank Screen Audio Burst; Change Ic6003 (mn15342 Vg1 Or Vgp). NATIONAL; NV250; Eject Button Pressed In Play, Ejects Without Going Into.; Replace Mode Select Switch. (to Be Done In Eject Mode And Align V Notch).. NATIONAL; NV250; Erratic Mechanical Behaviour; Mode Switch Vss0110. NATIONAL; NV250; Loads Then Unloads, Slow Rewind; Mode Switch Vss0110. NATIONAL; NV250; No Channel Display Or Select.; Change Ic7501 (mn14824vtu).. NATIONAL; NV250; No Colour In Playback; Q8001 2sd636 Or Ic801. NATIONAL; NV250; No Colour On Playback.; Change Q8001 (2sd636) Or Chroma Pack Vefc009.. NATIONAL; NV250; No Display; If Pin 9 Of Ic2005 Lower Than 2.8v Adjust Reset Vr7502.. NATIONAL; NV250; No Display; Check Adjustment Of Reset Vr7502. NATIONAL; NV250; No Display; R1103,q1101,d1105. NATIONAL; NV250; No Flashing Display; Change Q6001 (2sd636). NATIONAL; NV250; No Functions, On Clock.; Check R1103 O/c, Q1101 (2sa684) And D1105 (30v Zener). if 30 Volt Line Still Low Check Q7503 (2sd637r). NATIONAL; NV250; No Go No Display; Check R1103 O/c ,ic 6501 Mn14824vtu. NATIONAL; NV250; Not Detecting End Of Tape; Led Sensor D1501 Vjb00c48. NATIONAL; NV250; Picture Rolls, Loss Of Control Signal. Capstan Servo Roll.; Replace Faulty Capacitor, C2031 47uf 10v near Ic 2005 An6356n. NATIONAL; NV250; Play And Record Stops Intermittently.; Change Ic2005 (an6356n), Suspect Photo Interrupter (on2106). NATIONAL; NV250; Poor Lum In Ee & Colour Int; Ic301 Lumance Pack Vefy007. NATIONAL; NV250; Poor Remote Distance; D/j On C6504. NATIONAL; NV250; Power Up Capstan Rotates Then Turns Off; Ic6003 Ba6238 Also Check Worm Wheel On Eject Asy.. NATIONAL; NV250; Preset Stations Go Blank When Selecting But Retain Mem; Ic7503 Mn1228. NATIONAL; NV250; Preset Stations Inter. Go Blank Raster. Still In Memory. .; Replace Ic7503 (mn1228). NATIONAL; NV250; Remote Control Not Operating Over 1.5 Metres From Vcr.; C6504 On Display Pcb Possible Dry Joints.. NATIONAL; NV250; Stops In Pause Mode.; Faulty Mode Switch.. NATIONAL; NV250; Stops In Play And Record Int; Ic 2005 An6356 , Photo Interrupter On2106. NATIONAL; NV250; Switchs To Bypass Mode When Hot; 5v Line Low Q1003,q1004, Adjust Vr1001. NATIONAL; NV250; Unlaces And Slow Rewinds When Review Pressed.; Faulty Mode Switch.. NATIONAL; NV250; Vcr Not Switching Off.; Change Ic7504 (mn145532vtw). NATIONAL; NV250; Vtr/tv Sw Int Operation; D/j Pin 18 Ic 7502. NATIONAL; NV250; Wont Accept Tape. Wont Fully Eject Tape.; Replace Worm Wheel On Eject Assy.. NATIONAL; NV250; Wont Rec/play Int; 5v Supply Low (tp1001) Adjust Vr1001 Or Q1003,q1004. NATIONAL; NV280; After Goes Into Play Mode Stays Laced Up But Stops.; Replace Mode Select Switch. (to Be Done In Eject Mode And Align V Notch).. NATIONAL; NV280; Does Not Memorize Stations.; Replace Ic7503 (an1228).. NATIONAL; NV280; Drum &/or Capstan Varies Speed Or Fast; Ic2003 Mn6168vif. NATIONAL; NV280; Eject Button Pressed In Play, Ejects Without Going Into.; Replace Mode Select Switch. (to Be Done In Eject Mode And Align V Notch).. NATIONAL; NV280; Low Capstan Motor Torque.; Check Value Of R2029 (56k). NATIONAL; NV280; No Clock Or Channel Display; Ic7502 An5033. NATIONAL; NV280; No Clock Or Channel Display.; Change Ic7502 (an5033). NATIONAL; NV280; No Clock.; Q1101 Shorted & Zener Diode D1105 (30v) S/c.. NATIONAL; NV280; No Go; C1116 1000uf/10v. NATIONAL; NV280; No Or Low Capstan Speed; R2018 .82 Fusible (must Be Replaced). NATIONAL; NV280; No Power; R1103 O/c. Power Supply.. NATIONAL; NV280; Not Sensing End Of Tape Not Going Into Auto/rewind.; Q6006(2sd636) And And R6048,connecting Photo Sensor To Processor.. NATIONAL; NV280; Sometimes Catches Tapes. Capstan Motor Starts To Jerk And Stall Until Unit Shuts Off And Leaves Tape Out Of Cassette.; Mn6168vif Ic On Main Board Under Tuning Section.. NATIONAL; NV280; Supply Rails High; Replace C1110 C1111. NATIONAL; NV280; When Pause Selected In Playback, Tape Speed Increases.; Change Ic2003 (mn6168vif). NATIONAL; NV280; Wont Accept Tapes; Ic2 An3821 Tf1 Thermal Fuse. NATIONAL; NV280; Wont Remember Stations; Ic7503 Mn1228. Now you can find us faster by using our new Browser Toolbar ........................MORE.; Live Help close window.. Im ready to assist!. Testimonials. RESERVED FOR AD.. RESERVED01; RESERVED02.. Our Forum; Electronics Troubleshooting; Fletchermanuals.com; Fletchershopping.com; Our Blog; Site Map; Contact-Us. |  |  |
| News Letter. Schematic Request.; Sams-Club. Get Traffic.... <>... > National Repair Tips 1 2 [3] 4 Many More Tips In Ours Members Section! No Limitations For Full Repairtips Members! 17.. To Remove Limitations, Click HERE! PLEASE CHOOSE A BRAND:; Top of Form Bottom of Form.. Our Store!. National /Panasonic NV3... Series NATIONAL; NV300; Any Function Lights Up Power On Intermittently Changes Mode; Pin 8 Of Ic6005 Changes Amplitude, Normally 2.62v If Low Replace Ic6005 (upd4069ubc). NATIONAL; NV300; Blank Raster On Tuner.; Check C7009 (.01mfd).. NATIONAL; NV300; Buzzing In Sound With Low Sound On Recorded Programs.; Check 17v Rail If High (18.45) Look For D/j On D1013.. NATIONAL; NV300; Capstan Motor Speed Fast; Fault In Fg On C.Mtr. NATIONAL; NV300; Capstan Motor Wont Start When Loading Is Complete.; If Pin 6 Of Ic6001 Stays Low Check Mode Switch.. NATIONAL; NV300; Capstan Not Locking.; Bottom Board Plug Off.. NATIONAL; NV300; Capstan Slow Or No Go; C2049 10/16v. NATIONAL; NV300; Capstan Speed Faulty In Record Mode Play Mode Is Ok; Check Faulty C2034,0.1µf 100v. NATIONAL; NV300; Capstan Wont Lock.; C2026 .22uf Cap. NATIONAL; NV300; Clock Display Very Dim; D7511 Leaking. NATIONAL; NV300; Distorted Sound On Playback.; Change Q4014.. NATIONAL; NV300; Door Stays Up (ejects); Lever Broken. Q6025 To Q6030 Or Ic6005. NATIONAL; NV300; Drum Hunts. Poor Pic Like Heads; C2003 10/16v. NATIONAL; NV300; Drum Speed Int; C2022,c2023,c2026 2.2uf/50v. NATIONAL; NV300; Drum Wont Go. Play Mode Wont Load.; Check Q6002 (qsb641). NATIONAL; NV300; Function Buttons Not Operational Except Eject.; O/c Remote Jack.. NATIONAL; NV300; Goes In To Ff When Lid Closed; Plug From Reel Sensor Off. Lhs Rear 2pin. NATIONAL; NV300; Goes Into F.F. When Door Closed With No Tape In.; Bottom Board Plug Has Come Off.. NATIONAL; NV300; Hum In Video And Audio In Playback, Sound Wows, Hum In Ee; C1002 (2200 Uf) O/c. NATIONAL; NV300; Hum In Video/ee.Capstan Wont Lock.; C1002 2200uf Cap And C2026 .22uf Cap. NATIONAL; NV300; Int Failure To Record The Audio Track.; Replace Q4003, D4001, D4002, 2sb642rs/2sa564a.. NATIONAL; NV300; Int Leds Come On; Ic6005 4069. NATIONAL; NV300; Int Non Erasure Of Previous Sound Track. Chroma Sig. Remain; Look For D.J On Bias Osc Q4014.. NATIONAL; NV300; Int Wont Go Into Record, Or Stops Recording; Record Tab Switch Vsm0043. NATIONAL; NV300; Int Wont Start Drum; Replace C2026, 2027, 2028 .22µ 50v Caps. NATIONAL; NV300; Interm. Not Rec. Audio. No Audio Record.; Solder Wire From Connector Bp6009 Pin 1 Directly Onto Pcb.. NATIONAL; NV300; Mechanical Mode Switch Check.; Pins Of Bp6003 Should Go Low. stop Mode - 5, Laced Up Play Or Record - 1, F.Fwd. Rew. - 4, Record Pause - 3, Eject - 6.. NATIONAL; NV300; Missing 13v Rail; R1006 1O Fuseable. NATIONAL; NV300; Mode Sele. Switch Wont Return To Normal Pos.Cass Wont Engag; Check Q6010, Q6026, Q6028, Q6029 And R6023, R6024.. NATIONAL; NV300; No 18 V Capstan Motor; R1010 (3O3), D1007. NATIONAL; NV300; No Capstan 18v; Check R1010 (3.3ohm).. NATIONAL; NV300; No Clock; Ic7501 Mn1435vxb. NATIONAL; NV300; No Clock Or Channel Display.; Change Ic7502(an5033). NATIONAL; NV300; No Drum; Q6002 2sb641. NATIONAL; NV300; No Erase. Bias May Be Int.; C24 10 Vfd 16v Int.. NATIONAL; NV300; No Functions; Q 6008. NATIONAL; NV300; No Functions Ex Eject; Remote Control Socket Vej0229. NATIONAL; NV300; No Functions Operating; Change Q6008. NATIONAL; NV300; No Go.; Timer Record Switch.. NATIONAL; NV300; No Picture. Horizontal Lines, Wowing Sound.; Change Ic2003.. NATIONAL; NV300; No Play, Eject Etc.; Cass. "in" Switch (dirty).. NATIONAL; NV300; No Power. No Functions.; Check Remote Control Socket.. NATIONAL; NV300; No Reel Drive; Ic6006 µpc358, Motor Myn13v5l. NATIONAL; NV300; No Reel Motor Drive.; Pin 7 Of Ic6006 Should Read 14.2v Disconnect Pins 5 And 6 If Then Ok Replace Icupc358c.. NATIONAL; NV300; No Regulated 13v; Check Q6002 (qsb641). NATIONAL; NV300; No Tape Functions.; Check And Replace Load Motor Drive Transistors.. NATIONAL; NV300; No Tuner In Ee, Or Channel Indication; R7020 3O9 On Demodulator Pcb. NATIONAL; NV300; No Video In Ee Mode; Q703 2sd945. NATIONAL; NV300; No Video In Play; Q3022 2sd636. NATIONAL; NV300; No Video In Record; Q3017 2sb641. NATIONAL; NV300; No Video On Playback, Blank Raster Only.; Check Q3022 (2sd636).. NATIONAL; NV300; No Video On Tuner.; Check Q3017 (2sb641).. NATIONAL; NV300; Not Recording Or Erasing; Q6005 2sa719. NATIONAL; NV300; Not Tunning Stations; R7019, D7006. NATIONAL; NV300; Picture Fuzzy In Playback And Ee Through Rf Out Only. video Out O.K.; Replace Faulty Capacitors C9, C4, In Rf Modulator. (not Rf Box Where Leads Go In).. NATIONAL; NV300; Plays For A While Then Stops.; Mode Switch.. NATIONAL; NV300; Poor Or No Lum In Play; C3007 10µf/16v D3012. NATIONAL; NV300; Records Low Audio On New Tapes But Loud Audio On Old Tapes.; Replace Audio Control Stack.. NATIONAL; NV300; Rewinds And Ejects Intermittently.; Change Q6010 (2sd636). NATIONAL; NV300; Slow Rewind; Clean Supply Reel Shaft. NATIONAL; NV300; Snatching Tape In Review; Excessive Wear In Reel Idler. NATIONAL; NV300; Stays In The Eject Mode And Wont Accept Tapes.; Change/replace All Four Q6025, Q6028, Q6029, Q6030 And Ic6005 (upd4069ubc).. NATIONAL; NV300; Supply Voltage 15 V Low; C1003 3300uf25v. NATIONAL; NV300; Tape Speeds Increases In Pause Mode.; Change Qr201 (dtc124a) In Noise Advance Pack.. NATIONAL; NV300; Trouble With Tape In Review; Lower Drum Worn Sticking To Tape Polish. NATIONAL; NV300; Tuner Not Working, No Leds On Tuning.; Check R7020 (3.9ohm) Fuseible On Demodulator Board Also D7007.. NATIONAL; NV300; Tuner Not Working.; Check 30v Missing On Tuning Pots Because Ic7001 S/c.. NATIONAL; NV300; Vcr Stays In Rec Mode; Ic6005. NATIONAL; NV300; White Flickers On E-e And Playback.; Check 5v Line For Ripple (5mv P-p Normal). NATIONAL; NV300; White Lines In Play, Ee; Ripple In 5 Volt Rail. NATIONAL; NV300; Wont Go Into Mode Or Goes Berserk.; Ic6001 (mn1405) Faulty.. NATIONAL; NV300; Wont Go Into Mode Selected, Goes Berserk In Play Mode.; Check Faulty Ic6001 (mn1405vkf) Note These Last Three Letters Are Important, Must Replace Only With Original Part Being Replaced.. NATIONAL; NV300; Wont Lock In On Playback.; Check Ic3003 (an6327) In Luminance Channel.. NATIONAL; NV300; Wont Play. Tape Loaded. Capstan Not Functioning.; Replace Both Loading Belts.. NATIONAL; NV300; Wont Select Play, Eratic Tape Functions; R6056 680O Resistors Near Bj6007 Pin 2, Shb 5.24 V No Load.. NATIONAL; NV300; Wont Tune Channels.; R7019 O/c Tuner Board.. NATIONAL; NV3000; Capstan Motor On Ay All Times.; Check Q1016, 2sd946, Under Shield On Main Pcb. Cap. Stop. Low Should Be At Base Of Q1016.. NATIONAL; NV3000; Cuts Out In Play/fr/ff Ect; Electro Leaked. B/t P13 P14 Bottom Pcb Reel Pulse Low.. NATIONAL; NV3000; Dew Sensor On At All Times.; Suspect Ic 5, An6912, On Front System Control Pcb.. NATIONAL; NV3000; Dropout Compensator Not Operating,d/outs On Play.; Check Delay Line Dl3001, P.No. Vld0020. Rf Envelope Should Be Present At Both Input, Pin 1, And Output, Pin 3.. NATIONAL; NV3000; Drum Motor Fails To Start, Interm. Stops During Play/rec.; Go To The Edge Of The Main Pcb That Has Many Plugs Mounted Along It. Measure Ohms Between Pin 8 Of P9 And Pin 5 Of P15. This Should Measure Approximately 1.8 Ohms. if Measures High, Replace R2120 Mounted On Jumper Wire Between Pin 8 And Pin 5.. NATIONAL; NV3000; Int Lines In Pb. Drum Servo Fault; Dj On Leads To Cylinder Drive Pcb. NATIONAL; NV3000; Int No Colour In Pb; C8041 10µf/16v, Ic8004 An6342n. NATIONAL; NV3000; Int No Rewind; Q6552 T/up Sensor. NATIONAL; NV3000; Int Traking Problems; Check R2088 100k Preset Dj's. NATIONAL; NV3000; Lines In Picture. Mistraking.; Replace Back Tension Band If Faulty.. NATIONAL; NV3000; No Audio In Pb; Q4 2sd655 Leaky. NATIONAL; NV3000; No Audio In Record; C47 10µf/16v. NATIONAL; NV3000; No Capstan; Q2020 2sb641. NATIONAL; NV3000; No Loading Motor; Cap On Top Pcb 6800µf/16v Shorted. NATIONAL; NV3000; No Pic In Play, Distorted Video; C122 10µf/50v In Tuner. NATIONAL; NV3000; Play Audio Volume Faulty.; Check C22, 16v 10 µf And C19, 16v 10µf, In Audio Section Of Chroma Pcb.. NATIONAL; NV3000; Stops Int; Q6411 2sb641 On Load Mtr Pcb. NATIONAL; NV3000; Turns Off After Several Sec's; R6417 3O9 Fuse Res. Load Mtr Pcb. NATIONAL; NV333; 2sc2671 Transistor Used In The Booster Amp.,is Trouble .; Repair Kit, Part No. Vvk4059, Which Consists Of A 2sc2671 Transistor Plus Two Ma161c Diodes And Fitting Instructions.. NATIONAL; NV333; Buzzing Sound. E-e Normal.Recording Sound Very Low, Buzzing; Change D1013.. NATIONAL; NV333; Cant Tune Stations In, When In E-e Mode.; Check Tuner.. NATIONAL; NV333; Capstan Runs Too Fast.; Check Faulty Fg In Capstan Motor.. NATIONAL; NV333; Capstan Speed Varies, Sometimes Intermittently.; Monitor The Capstan Fg Pulses At Pin 5 Of P6001. If The Pulses Disappear Intermittently, Suspect A Faulty Fg Generator. This Is Within The Capstan Motor - Check The Interconnections First.. NATIONAL; NV333; Capstan Speed Wanders In Playback.; Check Whether Tracking Control R2064 O/c.. NATIONAL; NV333; Cass. Wont Eject, Or Carry Out Deck Functions.; Suspect The Mode Switch.. NATIONAL; NV333; Cassette Carriage Ascends Too Quickly.; Teeth In Damper Unit Worn. Part No. Vdg0100.. NATIONAL; NV333; Clock Failure With No Display.; Check 100 Ohm Safety Resistor (r7632 In Nv 366), Check Back Up Battery Is Not Leaking.. NATIONAL; NV333; Clock Not Lighting, Capstan Motor Not Turning.; No 18v Due To Q1002 (2sd973) Being O/c.. NATIONAL; NV333; Clock Wont Relight After Mains Supply Interruption.; Change Ic7501.. NATIONAL; NV333; Distorted Sound With Corrugated Verticals.; Thoroughly Clean The Erase Head - Replace If Necessary. Can Also Be Caused By A Worn Audio/control Head (vbr0050).. NATIONAL; NV333; Drum And Capstan Servos Pulsing And Out-of-sync.; Check R2017 Is Dirty.. NATIONAL; NV333; Excessive Noise In The Load And Unload Modes.; Replace Reel Idler Arm Vxl0997.. NATIONAL; NV333; Failure To Eject.; Check Switch, If Ok, Check C6011 (18µf) And Ic6002 (m53216p).. NATIONAL; NV333; Failure To Record The Sound.; Change The Str1096 Regulator Ic1001 In The Power Supply.. NATIONAL; NV333; Goes Into F.F. As Soon As The Try Is Closed.; Faulty Supply Reel Sensor.. NATIONAL; NV333; Head Switching Line Runs Through Picture.; 50hz Reference Switched On During Recorded Because Ic8004 (an6342n) Faulty.. NATIONAL; NV333; Hum On E-e Sound, Amplified To An Obliterating Buzz On Rec.; Check For Hum On The 17v Rail. If Present D1013 (1ss16) Faulty.. NATIONAL; NV333; Hum On Sound.; Ic4001 (ta7325p) Developing Hum Internally. Check The Inputs And Outputs With Scope.. NATIONAL; NV333; In Reverse Pic. Search Mode The Tape Loops Into Machine.; Faulty Head Drum. (the Engraved Tracks Wear Thin And The Tape Sticks).. NATIONAL; NV333; Inter. Head Switching Line Runs Through Pic.,drum Speed Var; Vss Signal Missing Due To A D/j At Pin 1 Of Ic301.. NATIONAL; NV333; Interm. Loss Of Colour Or Interm. Colour Bars On Screen.; Check If The Pulses At Tp8006 Are Less Than 6v P-p, Adjust R8036 To Correct.. NATIONAL; NV333; Intermittent Audio Erase.; Remove R4049 (4.7) In The Bias Oscillator Circuit And Replace With A Shorting Link.. NATIONAL; NV333; Intermittent Chroma On Record.; Connect A Fast-acting Diode (ma161 Or Ma162) Between Pins 11 And 12 Of Ic6003 On The Syscon Board, Cathode To Pin 11.. NATIONAL; NV333; Intermittent Failure To Eject Tape.; Check Key Scan Inhibited Due To C6011 (ecqv05184jz) Or Ic6002 (m53216p) Faulty.. NATIONAL; NV333; Intermittent Loss Of Capstan Lock In The Record Mode.; Suspect An Intermittent Fault In Ic2003 (upc1505c).. NATIONAL; NV333; Intermittently Cuts Out On Deck Functions.; The Counter Has A Hall-effect Sensor. Any Of The Following Can Cause The Complaint: The Reel Idler, Idler Arm, Counter Belt, A Tight Take-up Reel, The Hall-effect Device Faulty Or Too Far From The Magnet, Or A Defective Reel Motor.. NATIONAL; NV333; Knocking Noise On Fast Forward.; Noise Affected By Pressure On The Cassette. Lubricate The Spool Shafts.. NATIONAL; NV333; Lid Wont Stay Down. Seems To Be Permanently In Eject Mode.; Check The Operation Of The Cassette Down And Loading- end Switches, Mode Select Switch.. NATIONAL; NV333; Loading Motor Continues To Run. Stays In Loaded Condition.; A Very Common Cause Of This Problem Is Slippage Of Loading Belt 2. If Replacing This Doesn't Provide A Cure, Suspect Misadjustment Of (or Fault In) The Mode Switch. Its Mechanical Adjustment Is Very Critical.. NATIONAL; NV333; Loading Motor Stops During Loading.; Ic6005 (upd4069ubc) Probably Faulty. Also Check That C6008/9 Have Been Changed From 100pf To 300pf.. NATIONAL; NV333; Machine Intermit. Stops And Rewinds During P/back,or Rec.; Supply Phototransistor Q1503 (pn150nv) Faulty. Sometimes Ic6004 (an6912) Is Respnsible.. NATIONAL; NV333; Machine Wont Switch On.; Power On/off At Pin 3 Of P1003 Not Switching Due To A Faulty M53216p Chip (ic6002) In The Syscon Circuit.. NATIONAL; NV333; No Capstan Lock In Playback.; Check The D.C. Level At Pin 16 Of Ic2003 (upc1505c). If Low Suspect No 9v Supply As Q6003 (2sa719) In The Syscon Circuit Is O/c.. NATIONAL; NV333; No Capstan Motor Rotation.; Check Ry1 On The Servo Panel Faulty.. NATIONAL; NV333; No Chroma In The Record Mode.; If The Chroma Is Present At Tp8003 Check At Pin 8 Of Ic8001 (an6360). If The Chroma Is Missing Here And The D.C Level Is High (should Be Aspprox. 4.5v) D8005 (ma165) Is Probably Leaky - It Links Ic8001 To The Colour Killer In Ic8003.. NATIONAL; NV333; No Clock Display.; Ic7501 On The Front Panel Is Faulty. In The Nv333 It's Type Mn1435vxb. Getting The Suffix Right Is Vital.. NATIONAL; NV333; No Deck Functions.; Cassette Down Switch On Eject Latching Assembly Dirty Or Worn. Part No.Vsh0013.. NATIONAL; NV333; No Eject Damping - The Gear Teeth Inside Damper Unit Break.; Replace The Unit.. NATIONAL; NV333; No Field Lock In Playback.; Change Ic3003 (an6327). NATIONAL; NV333; No Field Sync On Playback.; Check The Signal At Pin 1 Of Ic3003 (an6327). If Missing, Suspect Ic3003. The Signal Should Be 2v P-p. If Less Than 1v P-p, Change Ic3003.. NATIONAL; NV333; No Functions Except Eject.; Check For Dirty Contacts On Thr Cassette-in Switch.. NATIONAL; NV333; No Go.; If Pin 8 (power Off) Of Ic6001 Is High, Q6010 (2sb641) Faulty.. NATIONAL; NV333; No Go.; No 9v And 6v Outputs From The Str1096 Chip Ic1001 At Pins 4 And 5 Respectively. Check Whether The Unregulated 15v Supply Is Present At Pin 1. If This Is Present And The Supplies Dont Return When The Loads Are Disconnected Ic1001 Is Faulty.. NATIONAL; NV333; No Playback Colour, Unlocked Colour Flickering Occasionally; If Pin 9 Is Linked To +5v And Colour Comes Back Ic3002 Faulty.. NATIONAL; NV333; No Power After 10 Sec., With No Loading.; No Loading Motor Drive As Ic6005 (upd4069ubc), Q6026 (2sb819) And Q6029 (2sd1051) Faulty. Change C6008 And C6009 (300pf)>. NATIONAL; NV333; No Reel Drive.; No Motor Drive As Q6022 (2sb641) And Q6023 (2sd12730) S/c.. NATIONAL; NV333; No Rewind Or Fast Forward With No Cue Or Review.; Check Q6023's Emitter For Dry Joint.. NATIONAL; NV333; No Rewind Or Fast Forward.; D1009 (ma165 Or 1ss133) Faulty.. NATIONAL; NV333; No Tuner Operation Or Channel Leds.; Check 3.9 , 0.5w Safety Resistor R7020 Is Probably O/c.. NATIONAL; NV333; No Tunning.; Tuning Voltage Missing R7019 (100 Ohm, 0.25w) O/c. The Upc574jk Stabilizer Ic7001 And Q7005 (2sb642) Probably S/c.. NATIONAL; NV333; Permanent Eject.; Change Q6028.. NATIONAL; NV333; Poor Field Sync On Playback Of Own Rec. Hum, Ripple On Edge; Check Hum On Thr 1.T. Lines Because One Of The Em1z Rectifier Diodes D1001-4 Is O/c.. NATIONAL; NV333; Poor Recording And Sound Coming Through From Other Channel.; Dry-joints On The Bias Oscillator Transistors Q4014.. NATIONAL; NV333; See Nv366. NATIONAL; NV333; Sluggish Rewind.; Check If The Reel Idler Is Ok Check The Supply Reel - it's Probably Tight On Its Shaft (lubricate As Necessary).. NATIONAL; NV333; Sometimes Wont Rec. Or Play. F.F. And Rew. Ok.; Set Up The Cam Gear And Action Gear As Per The Manual. Clean The Belt And The Pulley Wheels.. NATIONAL; NV333; Squeaking Capstan.; Put Into The Pause Mode To Confirm. This Fault Can Be Caused Either By Worn Bearings In The Capstan Motor Itself (max13v9lp), Or By A Dry Flywheel Bearing.. NATIONAL; NV333; Squeaking Noise Which Stops In Pause.; Noisy Capstan Motor.. NATIONAL; NV333; Squeaks When Loading Or Unloading.; Lubricate Loading Roller Bearings.. NATIONAL; NV333; Take-up Spool Rotates In The Stop Mode.; Check For Dirty Connections At The Supply Photosensor Connector P6010.. NATIONAL; NV333; Tape Is Not Reeled Back Into The Cassette During Unloading.; Replace The Reel Idler (vxp0401) And Check The State Of The Intermediate Geaar (vdg0017).. NATIONAL; NV333; Tape Loops Into The Machine In The Review Mode.; This Is Caused By Excessive Friction At The Drum. It's Sometimes Due To The Grooves On The Drum Being Worn, Or A Worn Lower Drum.. NATIONAL; NV333; Tape Snatching In The Review Mode.; Check Excessive Play In The Joints Of The Reel Idler Arm (vxl0997). If A Replacement Doesn't Provide A Cure, Fit A Washer On The Joint.. NATIONAL; NV333; Tape Will Not Unlace Or Will Not Complete Lacing.; Replace The Large Loading Belt And Check The Others.. NATIONAL; NV333; Tuning Drift.; Remove C7010 (0.22uf) On The Demodulator Pcb.. NATIONAL; NV333; Wavering Playback Sound.; Check For Worn Capstan Motor.. NATIONAL; NV333; When Cass Is Loaded The Machine Wont Respond To Any Command; Check The Cassette-down Switch Under The Main Board.. NATIONAL; NV333; When Review Is Selected The Machine Ejects The Cassette.; Check For Faulty Or Misaligned Mode Switch.. NATIONAL; NV333; Whine Or Whistle From Direct-drive Unit.; Check The Discharge Angle At The Top Of The Drum Was Worn. Sometimes The Dd Motor Bearings Become Noisy.. NATIONAL; NV333; Won't Switch Off, Even When Timer Button Is Pressed.; The 2.5a Fuse On The Mains Transformer Pcb Is O/c. Change Ic6002 (m53216p).. NATIONAL; NV333; Won't Tune In.; No 30v Supply At Pin 10 Of P7005 Because D7005 (ma26wcb) In The 30v Regulator Circuit On The Demodulator Pcb Is S/c.. NATIONAL; NV333; Wont Eject And No Other Functions Work.; Check Eject Switch Sw6501.. NATIONAL; NV333; Wont Erase Prev. Sound Track. Pic. Has Chroma Washover.; Check For No Erase Bias, Probanly Because Of D/j's On Q4014 And T4001 On The Audio Panel. If The Problem Persists, Or Low Bias Seems To Be Incurable, Link Across R4049 (4.7mfd) In Q4014's Emitter Circuit.. NATIONAL; NV333; Wont Lock In The Playback Mode.; Q6003 O/c.. NATIONAL; NV333; Wont Rec In Colour. Replays Prerecorded Tapes In Colour.; D8005 (ma165) Which Turned Out To Be Leaky.. NATIONAL; NV366; Bottom Half Of Pic., Covered In White Noise Spots, Flicker.; Replace Rl3501 & Rl3502.. NATIONAL; NV366; Cass. Stuck, Only Clock, Programmer Work.; Sitch Ing Off The Power For A Long Time Restores Normal Operation Until Next Lock-up. Check The Syscon Microcomputer Chip That Its 5v Supply (pin 39) Is Present And Correct, With No Ripple/hash. If Ok Change (mn1405vkk).. NATIONAL; NV366; Channel Change Faulty, Wrong Led Lights Up.More When Cold.; Replace Q7553 (2sd637) With Improved Typ[e 2sd637s Or T Or Type 2sc1685s Or T. In Addition Fit A 2.2k Resistor In Unused Position R7565.. NATIONAL; NV366; Check 3.3mfd 2.0v (3 Off).; Leak Goo Into Board, Sparks When Hit With Soldering Iron. Check Also R1101. .12 Ohms For O/c.. NATIONAL; NV366; Cue/review Modes Are Released And Returns To Play.; Check Dirty Contacts On Switching Relays Rl3501/2. Replace Both (part No. Vsy2029). NATIONAL; NV366; Interm. Noisy Picture,tracking Control Has No Effect.; Replace Faulty Switching Relays.. NATIONAL; NV366; Intermittently Incorrect Capstan Speed.; Check The Capstan Fg Signal At Tp2007. When Motor Is Gently Tapped The Speed Corrects Itself. Change Motor - (the Capstan Fg Generator Is Built Into The Motor.). NATIONAL; NV366; No Clock Display.; Ic7501 On The Front Panel Is Faulty In The Nv366, It Is A Mn1455bub. Getting The Suffix Right Is Vital.. NATIONAL; NV366; Pic. Noisy After The Still.; Check Dirty Contacts On Switching Relays Rl3501/2. Replace Both (part No. Vsy2029). NATIONAL; NV366; Saturated Colours From Pre- Recorded Tapes Only.; Check Back Tension Should Be 25/30 Grams.. NATIONAL; NV366; See Nv333. NATIONAL; NV366; Still Mode Pic. Has Black Spikes.; Replace C3529 And C3530 With 40pf Type Ecv1zw40x53n Capacitors And Change R3522 To 100. Fit A 22pf, 50v Capacitor (c3535) Across Pins 1 And 2 Of P002. Readjust The Head Amplifier Peak Frequency (tp3508) To 4.9 Mhz Instead Of 4.5mhz.. NATIONAL; NV366; This Machine Doesn't Like Hg Tapes.; Machine Will Squeal In Record And Playback. E180 Tapes Work Ok.. Now you can find us faster by using our new Browser Toolbar ........................MORE.; Live Help close window.. Im ready to assist!. Testimonials. RESERVED FOR AD.. RESERVED01; RESERVED02.. Our Forum; Electronics Troubleshooting; Fletchermanuals.com; Fletchershopping.com; Our Blog; Site Map; Contact-Us. |  |  |
| News Letter. Schematic Request.; Sams-Club. Get Traffic.... <>... > National Repair Tips 1 2 3 [4] Many More Tips In Ours Members Section! No Limitations For Full Repairtips Members! 19.. To Remove Limitations, Click HERE! PLEASE CHOOSE A BRAND:; Top of Form Bottom of Form.. Our Store!. NATIONAL NVG.. NATIONAL; NVAM1; Switch mode Supply Starts Then Stops After 2 Seconds.; D4 Leaky. NATIONAL; NVAM1; Switchmode Supply Won't Start.; R13 O/c Or Hr. NATIONAL; NVAM3; Switchmode Supply Starts Then Stops After 2 Seconds.; D4 Leaky. NATIONAL; NVAM3; Switchmode Supply Won't Start.; R13 O/c Or Hr. NATIONAL; NVG10; Colour Drop-outs And Lines.; C1002 100uf/35v. NATIONAL; NVG10; No Channel Select, Number Indicator, Or Preset; Check For 30v At Pin 8 Of Plug Ja. If Missing, check R7572, 47k Ohms, In The Tuner And Demodulator Section Of The Main Pcb.. NATIONAL; NVG10; See Nvg7. NATIONAL; NVG10; Vtr On/off Faulty. Red Vtr Indicator Flashes When Vtr Off.; Check For Open Circuit D1006, Ma165,near Ic 1 In Power Section Of Main Pcb.. NATIONAL; NVG11; Breaking Tapes.; Faulty Hall Effect I.C. On Stator.. NATIONAL; NVG11; Tuning Problems With E7 Serial No Prefix.; Change Ic7551 O7555 Q7551 Q7552 Because I.C. Legs Bend Over And S/c's Are Produced.. NATIONAL; NVG12; Damaging Edge Of Tape In Play Or Rec Modes.; Adjust P5 Post.. NATIONAL; NVG12; Mistracking. Noise Bars In Picture.; Replace Pinch Roller With New Part Number Vxl17433.. NATIONAL; NVG12; See Nvg7. NATIONAL; NVG16; No Display, No Functions ,operate Light On; Dry Joints To Q15 Drum Heater. (series Pass For + 6v Reg.). NATIONAL; NVG18; See Nvg7. NATIONAL; NVG21; No Or Intermittent Power; C25 & C18 (47uf 50v) O/cct In Power Supply. Low +23 To Regulator Ic.. NATIONAL; NVG21; Red Vtr Indicator On,wont Turn Off,no Clock.; Check Volts On Pin 10 Of Ic 7551, An5034,in Ch. Select Section Of Main Pcb At Rear Of Board. Section Ref.No. 7500 Series. Pin 10, Pclr Line Goes To Pin 11 Of Ic 7502, Mn1220 On Display Pcb.. NATIONAL; NVG21A; No Power .Chopper Faulty And Clicking Noise.; Insulation Between Chopper And Power Box In Wrong Position.. NATIONAL; NVG22; Vtr Operate Light On. Vcr Function May Still Work. Clock Shuts Down. Noise On Pins 2,3,4, Of P1001(socket On Switch Mode Pwr Supply).; Change C1023 10v 1000uf Capacitor.. NATIONAL; NVG25B; Interm. Sound And Pic. Dropouts ,creasing Bottom Of Tape; Pinch Roller Barrel Shaped.See Page 61 Of V.C.R. Technical Service Information For Diagram.. NATIONAL; NVG25B; Interm. Sound And Pic. Dropouts,creasing Bottom Of Tape.; Pinch Roller Barrel Shaped. Check If The Creasing Occurrs When The Tape Moves In The Fully Laced Position.. NATIONAL; NVG25B; No Video On Monitor In E-e Or Play.; Check If Q3501 (2sc2206) O/c (2sd636 Can Be Subs.) D3502 (ma27w) O/c.. NATIONAL; NVG30; Poor Colour Or Flashing In Play.; C2 In Power Supply Near Regulator.. NATIONAL; NVG40; No Power Dead.; Replaced Fuse F1001.. NATIONAL; NVG40; Poor Capstan Torque, Causing Motor To Stall During Changes Of Cassette Functions And Vtr To Turn Off Intermittently; Suspect Faulty Ic2501, Ba6432s On Sub Main Cba. NATIONAL; NVG40; When Tape Inserted At Very Beginning. Machine Ejects Tape.; Replaced Both End Sensors.. NATIONAL; NVG500; Power Supply Fails To Start Or Restart After Power Interruption.; C1003 1uf/350v Cap O/c.. NATIONAL; NVG7; Int Failure To Retract Tape When Unloading.; Faulty Mode Select Switch Pt. No. Vss0110.. NATIONAL; NVG7; Int Stop In Play Mode Or Rec. Or No Take Up.; Faulty Mode Select Switch Pt. No. Vss0110.. NATIONAL; NVG7; No Power Up At All.Reset Being Held High.; Vr7502 Out Of Tolerance. Ajust Until Power Up.. NATIONAL; NVG7; Unit Accepts Tape Then Rejects It.; Check End Sensors Aren't Leaky R6009 For Shorts To Surrounding Circuit.. NATIONAL; NVG9; Drum And Capstan Motors Not Locking.; Main Regulator Ic Stk5331..12v Line Faulty.. NATIONAL; NVG9; Wont Memorize Tv Stations.; Replace Ic 7502 And Check Cap 7510 10µ50v. NATIONAL; NVH65; Plays Black And White. Capstan Speed Varies In Play.; C1023 (1000uf 25v) Low Or O/cct. Excessive Ripple. NATIONAL; NVH70; Memory Of Preset Tv Stations Faulty.; Suspect Faulty Ic 7502, Mn1220.. NATIONAL; NVH70; Memory Of Tv Stations Faulty.; Suspect Faulty Ic7502, Mn 1220.. NATIONAL; NVH75; Drum Servo Pulses In Speed.; C10028 (330uf/35v) Faulty. Noisy +5 And +6v Power Rails.. NATIONAL; NVH75; No Drum Or Capstan Lock, Interference On Ee Picture; Replace C1028 330µ25v (use 105 Deg Cap) In Psu, It May Read Ok. NATIONAL; NVH75; No Power; Check Q1006, 2sc1384 (r,s) In Power Cba Unit. Check Diode D1008, Ma185.. NATIONAL; NVM1; Picture Goes Green Then No Colour.; Check Pin 12 Of Ic401 Should Be 2.5v If Higher Replace Ic401.. NATIONAL; NVM5; Noisy Audio On Playback Of Own Recordings.; Check Isolate Dc Lead Rubbing Against Drum Motor. (picking Up Noise.). NATIONAL; NVM5; Screen Is Brilliantly Lit Up.; Check -5 Line Drive From Transformer Power Pack (diode Goes O/c). NATIONAL; NVM5; Unit Turns On By It Self After A While. Also Wont Turn Off Sometimes..; Operate Switch Faulty... NATIONAL; NVM5; Unloading Is Complete.; Because Pinch Roller Returns In Front Of Drum Exit Loading Guide, Remove Excess Grease On Cam Gear, Clean Hardened Grease Out Of Teeth Of Loading Gear.. NATIONAL; NVM7; Clock And Calender Not Functioning.; Check For D/j's On Pin 31 Of Ic1.. NATIONAL; NVV10; Display Flickers.; Caps C111 & C108 330 µ & 100 µ Repalce.. NATIONAL; NVV10; No Go; Caps In Power Supply, C106 2200 µ 16 V Cap Leakie. NATIONAL; NVV10; See Nv100. NATIONAL; VWA18; No Operation Or Power.; Internal Fuse In Primary Of Mains Transformer May Go Open Circuit. Take Leads 2 From Fo1 To Bluem Wire, Pin 8 Of Voltage Select Switch, Sw02.. NATIONAL; VWA18; No Operation Or Power.; Check For Short Circuit Diode. Diode Has No Reference Or Location Number And Is Not Shown In Service Manual Diagram. Located Next To C13 Near Large D11.. NATIONAL; VWA18; Unit Stays In Tv Mode Wont Switch To Vtr Mode.; Check For Open Circuit R11, 39 Ohms, On Front Operation Pcb.. NATIONAL; VWCG1; See Nv-m7. NATIONAL; VWET180; No Clock Display. Poss. Tuner & Vtr/tv Indicators On.; Suspect Ic6723, Mn1512vtp, On Main Timer Pcb.. NATIONAL; VWET180; Switch Mode Noise Audible; C105 1000uf 16v O/c Or Low Capacitance.. NATIONAL; WAM7A; Not Charging Batteries; Q101 (2sb952) Dry Joints On Base And Collector Also Broken Base Track. Ilastic Transistor Case to Dampen Movement.. Now you can find us faster by using our new Browser Toolbar ........................MORE.; Live Help close window.. Im ready to assist!. Testimonials. RESERVED FOR AD.. RESERVED01; RESERVED02.. Our Forum; Electronics Troubleshooting; Fletchermanuals.com; Fletchershopping.com; Our Blog; Site Map; Contact-Us. |  |  |